Science Question and Answer #14

G. David Nordley

Q: Can you please talk about very large structures?  What are orbital tethers, artificial planetary rings, Ringworld-type things, and Dyson spheres?

A:  An orbital tether is simply a string, or cable, in orbit.  Artificial planetary rings are generally solid structures circling planets that can serve as habitat area.  Larry Niven's prototypical Ringworld is a ring around a star spinning fast enough, with walls high enough to maintain a breathable atmosphere without need of a roof.  There is a continuous range of smaller variants that merges into the range of rotating space habitats and classic rotating space stations.   A Dyson sphere surrounds an entire star, enabling the civilization that builds it to capture and use (to the extent that the laws of thermodynamics allow) all the energy put out by the star.

We don't have space here to assess all of them with any rigor, but I'll try to say a few words on each and point out some problem areas for writers thinking to use them.  This issue, we'll discuss the tensional structures; tethers and ringworlds.  Next time, we'll look at planetary rings, which can be tensional or compressional, and Dyson spheres.

Space Tethers:

To be used for something, a space tether needs to be put in tension, i.e. pulled taut.  There are two main ways of doing this; 1) putting it in orbit and allowing tidal forces to pull it in and out stabilizing it in a "gravity gradient" mode (most space tethers flown to date have used this mode), and 2) rotating it so centrifugal forces (okay, purists, "centripetal reaction forces") supply the tension.

In orbiting "gravity gradient" tethers, the center of mass of the tether moves at orbital velocity.  Since orbital velocity is higher the closer one is to the planet, the bottom half ("arm") of the tether is at less than orbital velocity for its altitude and tries to fall down.  The top arm moves at more than orbital velocity than its altitude; it tries to fly out.  Thus the tether is stretched with one end pointed toward the Earth and the other away from the Earth.

If an elevator car is placed on the tether, it can catch a payload at less than orbital velocity and take it up to where it is at more than orbital velocity.  The energy gained by the payload is lost by the tether and must be made up to keep the tether in the same orbit.  This can be done in several ways, one being to bring payloads from deep space to suborbital trajectories, another being to run a current through the tether, allowing it to push against the Earth's magnetic field like the rotor in an electric motor.

The orbital towers described by Clarke, Sheffield, and others are a special case of a gravity gradient tether; their centers of mass have an orbital period equal to the Earth's rotational period, like a communications satellite, so their inner tips could extend all the way to the surface.  These are very long structures--the inner arm is about 42,000 km from the center of mass and the outer arm could be an equal distance out, more, or less, depending on its mass.  Because each section of tether must support the weight of everything below it, they are tapered with the thickest part being at the center of mass.

The tip of an orbital tower does not have to be exactly at the equator (Clarke did not have to move Sri Lanka), though moving the tip away from directly down will increase tension somewhat.  As a thought experiment, imagine standing at the south pole and swinging an infinitely strong string with weight on the end around in a circle.  Now, taking care to keep it swinging fast enough to keep it off the ice, let the string out gradually.  Both the velocity and the period of the swing increase.   Keep doing this (in your mind) until the weighted end is beyond synchronous orbit and it takes all day to swing around you.  The point is that enough tension can counter any perturbative, sideways forces, leaving a curved, but usable, tether.

Can something like that be built around Earth?  Maybe.  The best plausible materials approach the strength of carbon bonds in graphite or bucky tubes--something like 4.2 E10 Pa [The "E" stands for "times ten to the..." so that "1E6" would be "1 times ten to the sixth," or 1 million. We use the "E" notation in sympathy for our editor :-).  Pa stands for Pascal; one Pa is one Newton per square meter.]  Orbital towers of this material would still need to be several hundred times as wide at geosynchronous altitude as at Earth and tens of thousands of times as heavy as the payloads they lift.

This would be a very daunting project to launch from Earth--though one could imagine a "boot strap" operation using rotating tethers and electromagnetic orbit raising.  However, given access to space resources, zero gravity manufacturing facilities and robotic labor, it begins to look more plausible.  Making one at Mars would be easier.

Rotating tethers fly through space like an Argentine bolo, generally (but not necessarily) rotating in the plane of their orbit.  If orbiting a planet and rotating in the same sense as the orbit, the tip of such a tether swings planetward backward against its orbital velocity, then up and starward faster than orbital velocity.

A rotating string tether in a three-day elliptical orbit  with a three-kilometer-per-second tip speed could pick up a payload at slightly less than orbital velocity and throw it to Mars to arrive in three to four months during a period of a hundred days or so as Earth approaches its conjunction with Mars.  Free space rotating tethers could catch payloads going one way and release them going in another direction.  These systems are tapered and the amount of tether material, and thus their mass, depends on strength and tip speed.

A free space tether is, essentially, a rocket; when mass released at the end of a tether goes one way, the tether goes the other way; rotating tethers could be attached to asteroids to move them.

Orbiting tethers designed to throw payloads should be much more massive than their payloads, or the act of throwing the payload would deorbit the tether!.  The tether system to perform the Mars mission would have a mass of roughly a hundred times the mass of the payloads it throws. The orbital energy lost in throwing the payload must be made up with propulsion or incoming payloads, as with the gravity gradient tethers.

Niven Ringworlds:

Structurally, most of the rotating space station, colonies, ringworlds, etc., are simply hoops of tether material with stuff built on them.  They are in tension and would fly apart unless held together.  Those whose calculus is not too rusty can derive the formula for the tension by mentally cutting the hoop in half and adding up (integrate) all the forces perpendicular to that mental division.  Assuming I've done that correctly, after substitutions and cancellations, the tensile or compressive strength required for a ringworld is simply:

(1) St = (X+1)*a*d*r

where a is the net acceleration (centrifugal - gravity), d is the density, and r is the radius of the ring, and X is the fraction of non-load-bearing mass (dirt, plants, and oceans).

Niven's Ringworld has an acceleration of 9.73 m/s and a radius of 1.53 E11 m (about 1.02 AU).  "Shadow" squares orbiting further in provided a night-day cycle.  If you think of it as a bicycle tire, it had"sidewalls" 1600 km high to keep in its atmosphere. I don't think that alone would be enough; the ultraviolet light of its sunlike star would split off the hydrogen in its water vapor and the solar wind would blow the hydrogen away, leaving it dried out like Venus or Mars--but see below.

If we assume its mass is equally divided between its bio-"sphere" and its structural material (called "scrith"), then X = 1.  If we then assume that both have a density of about 2,000 kg per cubic meter, we find that the strength of scrith is:

St = 2*9.73*2000*1.53E11= 5.7E15 Pa.

This is about six orders of magnitude stronger than the best tensional material, carbon bonds, can give us.  It is said scrith approximates nuclear bond strength, but the nuclear force is short range; with nuclear force strength comes nuclear densities which are even more than a million times worse.

Is "scrith" beyond the bounds of the possible as well as just beyond the bounds of the known?  I haven't investigated all the possibilities well enough to say that for sure, but my gut feeling is that  we would have to be talking about nuclear-density substances that are either inherently unstable or too massive, or both.

How big a ringworld can be built with physically possible materials?  If I solve the above equation for r and set X = 0 (making the dirt, air and rocks a negligible coating on the structural material, I get:

(2)	r = (St/d)/a.

That's really simple. St/d is a good measure of material capabilities; the stronger the material, the bigger the ringworld.  The lower the gravity, the bigger the ringworld.    In the table below, I compare the ringworld building ability of some tensional material currently available in bulk (such as spectra, used in fishing line) and carbon bond material.  (Note, the ratio, St/d, below embodies other assumptions, and will vary among sources).

material	St/d	Maximum ringworld radii in km

*	 (for 1 g)	(for 1/6 g)

steel	0.423	38	249

spectra	1.818	165	1070

carbon	21.000	1,909	12,350

*in meganewton-meters per kilogram (MN-m/kg).

I've looked at 1/6 g (lunar gravity) ringworlds as well as 1g, because they can be bigger and Titan shows us that you can have a dense atmosphere at 1/7 g as long as you keep the top of it cold enough.  A ringworld is ideal for that; run a current around it and it will have a magnetic field that will keep away the charged particles that heat up the top of atmospheres.  Water should then freeze out below the ozone layer and hydrogen leakage would be minimized.  Doing that, one should be able to use rim-walls "only" a few hundred kilometers tall.

A ring-world/star combination is gravitationally unstable in the radial direction.  Move it off center and the pull on one side exceeds the pull on the other side, and it moves more off center until disaster.  Niven's Ringworld needed active controls.   I will note, however, that light pressure behaves the opposite of gravity and try to push the ring as far from the star as possible. I'm not sure which force wins with very small displacements.   Also, stars have magnetic fields and charged particle winds which could interact with a magnetic field generated by currents running around the ring. (Having a flare interact with the surface itself, while dramatic, had unfortunate  consequences...)

I haven't taken self-gravity into consideration, but a total mass about equal to Jupiter spread around a 1AU ring isn't going to significantly reduce centrifugal force.

Orbital Ringworlds

There is another, smaller, ring world from the gaming field called "Halo."   It has a diameter of 3 million kilometers,  about 1% of Ringworld, but still too big for plausible materials; it needs stuff with a strength of about 26 trillion newtons per square meter, "only" a few thousandths of "scrith."  It  orbits (tilted) outside its star so it doesn't need an attitude control system, has about rotates about once a day and has about 1 g surface gravity (about 0.8, with those numbers).  One web source (Ref.2) implies its mass is 1/1500 that of Earth and 20 times Earth's surface area.

The arguably buildable lunar gravity ringworld in the table above would have a "day" of about 4.7 hours.  A free-space ring world could be augmented with a set of rotating string tethers as spokes, but it wouldn't help enough to make a 24 hour-day ringworld; for days that long, use mirrors and/or shadow squares.